| Direction relative to device | Source | Destination | Protocol | App Protocol | Port | Reflexive | PHI | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bi-directional | DAS, Workstations | App Server | TCP | HTTPS | 443 |  | Yes/ Secure | All Communications with the Repository back-end |
| Bi-directional | DAS | App Server | TCP | Proprietary | 8000 |  | No | Sends data acquired from the Physiological monitors over the TCP connection |
| Bi-directional | Workstations | App Server | TCP | Proprietary | 8002 |  | No | Sends data acquired from the Physiological monitors over the TCP connection |
| Bi-directional | Workstations | App Server | TCP | Proprietary | 8004 (See 8014) |  | Yes | Send / Receive Messages from Clients to be broadcast to Workstations |
| Bi-directional | Workstations | App Server | TCP | HTTP | 8005 |  | Yes | Only used by the Newborn Module |
| Bi-directional | Workstations | App Server | TCP | Proprietary | 8006 |  | Yes / Secured | Only used by the Newborn Module |
| Bi-directional | Workstations | App Server | TCP | Proprietary | 8007 |  | Yes / Secured | Only used by the Newborn Module |
| Bi-directional | Workstations | App Server | TCP | HTTPS | 8008 |  | Yes / Secured | Only used by the Newborn Module |
| Bi-directional | Workstations | App Server | TCP | Proprietary | 8009 |  | No | Used in enabling Single-Sign-On |
| Bi-directional | Network Redirectors | App Server | TCP | Proprietary | 8012 |  | No | ONLY IF DIGI SERIAL SERVERS ARE USED Sends data acquired from the Physiological monitors over the TCP connection |
| Bi-directional | Workstations | App Server | TCP | Proprietary | 8014 |  | Yes / Encrypted | Send / Receive Messages from Clients to be broadcast to Workstations |
| Bi-directional | Decision Support Server | App Server | TCP | Proprietary | 7101 |  | Yes / Encrypted | Data communication to the decision support modules |
| Bi-directional | Workstations | Decision Support Server | TCP | HTTPS | 7802 |  | Yes / Encrypted | Patterns service, Tracings interpretation data feed |
| Bi-directional | Workstations | Decision Support Server | TCP | HTTPS | 7803 |  | Yes / Encrypted | Plugins service, Cues algorithm data feed |
| Bi-directional | Workstations | Decision Support Server | TCP | HTTPS | 8081 |  | Yes / Secured | Data feed from decision support modules |
| Bi-directional | Workstations | Decision Support Server | TCP | SignalR | 8082 |  | No / Secured | Live data feed from decision support modules |
| Bi-directional | Workstations | Decision Support Server | TCP | HTTPS | 8083 |  | Yes / Secured | Vigilance dashboard – single patient browser based app |
| Bi-directional | Workstations | Decision Support Server | TCP | HTTPS | 8084 |  | Yes / Secured | Vigilance HUB multi-patient browser based app |
| Bi-directional | Decision Support Server | Decision Support Server | TCP | Net.tcp | 8201 |  | Yes / Secured | HUB DataCollector, patient information & Cues algorithm data feed |
| Bi-directional | Decision Support Server | Decision Support Server | TCP | net.tcp | 8203 |  | Yes / Secured | HUB DBService, patient information & Cues algorithm data feed |